# Routing and backflush setup

# QAD EE 2016

Table of Contents

Objective	3

Process Steps	3

Employee Create/ Employee Modify: 36.1.7.1 / 36.1.7.2	3

Department Maintenance: 14.1	4

Work Center Maintenance: 14.5	4

Standard Operation Maintenance: 14.9	5

Routing Maintenance: 14.13.1	6

Item Planning Maintenance: 1.4.7/ 1.4.17 for multi-site - IMPORTANT	6

Production line maintenance: 18.22.1.1 - IMPORTANT	7

Manual backflush transaction: 18.22.13	8

# Objective

In order to successfully complete backflush transaction several setup steps need to be made.

# Process Steps

To perform backflush it is required to create Employee and Department.

## Employee Create/ Employee Modify: 36.1.7.1 / 36.1.7.2

Here it is enough to create only one employee. No other fields are required to be filled.

Should be created already by G2 team

## Department Maintenance: 14.1

Here it is enough to create one department for BF only. No other fields are required to be filled.

Should be created already by G2 team

## Work Center Maintenance: 14.5

For backflush separate work center has to be created: BK

To this work center you need to link previously created Department (14.1)

Should be created already by G2 team

## Standard Operation Maintenance: 14.9

One standard operation must be created in the system with code 999.

It must me setup as 100% Yield Percentage and checked as Milestone Operation.

Also previously created Work Center has to be linked to 999 operation

Should be created already by G2 team

## Routing Maintenance: 14.13.1

When creating default routing following data should be setup:

- Standard Operation should be assigned

- Work Center should be assigned

- Milestone Operation should be checked

- Yield Percent should be set 100%

Should be created already by G2 team

## Item Planning Maintenance: 1.4.7/ 1.4.17 for multi-site - IMPORTANT

The most important parameters here to setup are:

- Routing Code (NEW Requirement) – must be assigned to each part that is being produced.

## Production line maintenance: 18.22.1.1 - IMPORTANT

All manufactured parts must be assigned to production line via 18.22.1.1 program.

## Manual backflush transaction: 18.22.13

When you do manual backflush in QAD EE 2016 following data must be entered:

- Employee: created on previous step

- Site: choose relevant site for this part

- Item number: Item that you want to backflush

- Standard operation: created on previous step

- Routing code (default): created on previous step

- Location (default): location to which manufactured part will be received

- Work center (default): created on previous step

- Qty processed: how many pieces you want to backflush